# Supplementary Material A

# INTRODUCTION

This supplementary section provides a complete step-by-step description of the QGIS workflow used to produce the Composite Exposure Index (CEI). The protocol is intended to ensure methodological transparency and allow independent reproduction of all GIS procedures described in the manuscript.

This supplementary material provides a detailed description of the geospatial workflow developed to calculate the Composite Exposure Index (CEI) presented in the manuscript. The purpose of this document is to ensure methodological transparency and facilitate reproducibility of the proposed framework by providing a comprehensive description of all processing steps, input datasets, software environments, spatial analyses, and model calculations.

The proposed framework integrates three complementary spatial components representing different aspects of recreational boating-related environmental pressure:

- Traffic intensity, derived from multi-year Automatic Identification System (AIS) vessel density data

- Ecological sensitivity, represented by the proportional coverage of Natura 2000 marine protected areas within each analysis grid cell

- Bathymetric susceptibility, expressed through a shallow-water indicator derived from GEBCO bathymetric data.

These components are combined within a regular 2 × 2 km spatial grid using a weighted linear combination to produce the Composite Exposure Index (CEI). The framework was designed as a transparent and transferable methodology that relies exclusively on publicly available spatial datasets and standard geographic information system (GIS) operations.

The complete workflow was implemented primarily in QGIS using standard geoprocessing tools, while statistical analyses and sensitivity analyses were performed in R. No proprietary algorithms or custom-developed software were required, enabling straightforward implementation by researchers and environmental managers using commonly available open-source software.

The complete R workflow, including scripts for data transformation, CEI calculations, classification procedures, sensitivity analyses, and figure production, is provided in Supplementary Material B.

# DATA SOURCES AND SOFTWARE

### Input datasets

### IHO Adriatic Sea boundary

The study area was defined according to the official limits of the Adriatic Sea established by the International Hydrographic Organization (IHO, 1953). The digital representation of the Adriatic Sea boundary was obtained from the Marine Regions database maintained by the Flanders Marine Institute (VLIZ).

Source:

Marine Regions – Adriatic Sea (ID: 3314)

Dataset used: Marine Regions Gazetteer polygon layer.

### Recreational vessel density (AIS)

Recreational boating activity was represented using vessel density products available through the European Marine Observation and Data Network (EMODnet Human Activities).

Monthly AIS vessel density rasters for the period 2017–2023 were downloaded for the following categories:

- Sailing vessels (code 04)

- Pleasure craft (code 05)

The original raster resolution is 1 × 1 km, and values represent monthly sailing hours per km².

Source:

EMODnet product catalogue

### Natura 2000 marine protected areas

The Natura 2000 dataset was obtained from the European Environment Agency (EEA) as polygon shapefiles (version end 2024), representing the official boundaries of Special Areas of Conservation (SACs), Sites of Community Importance (SCIs), and Special Protection Areas (SPAs) within the European Natura 2000 network.

Only marine Natura 2000 sites located within the Adriatic Sea were retained for the analysis.

Source: European Environment Agency – Natura 2000 spatial dataset

Dataset format used:  ESRI Shapefile.

### Bathymetric data

Bathymetric information was obtained from the General Bathymetric Chart of the Oceans (GEBCO) 2023 global bathymetric grid.

The dataset provides global bathymetry at a spatial resolution of 15 arc-seconds.

Source: GEBCO Web Services (WMS) ¸

Table 1. Table S1. Spatial datasets used in the CEI framework.

| Dataset | Provider | Format | Resolution | URL |
| --- | --- | --- | --- | --- |
| Adriatic Sea boundary | Marine Regions (VLIZ) | Polygon | – | Marine Regions – Adriatic Sea (ID: 3314) |
| Recreational vessel density (AIS) | EMODnet Human Activities | GeoTIFF | 1 km | EMODnet product catalogue |
| Natura 2000 polygons (version end 2024) | European Environment Agency | ESRI Shapefile (.shp) | Vector | European Environment Agency – Natura 2000 spatial dataset |
| Bathymetry | GEBCO | GeoTIFF | 15 arc-sec | GEBCO Web Services (WMS) |

### Spatial reference system

All spatial analyses were performed using the European Terrestrial Reference System 1989 – Lambert Azimuthal Equal Area projection (ETRS89 / LAEA Europe, EPSG:3035).

This equal-area projection was selected because it enables accurate calculations of area, spatial overlap, and zonal statistics across European marine regions while maintaining consistency among all spatial datasets.

Before analysis, all raster and vector datasets were transformed into this coordinate reference system. Raster datasets were additionally aligned to identical cell size, spatial extent, and pixel origin to ensure pixel-by-pixel compatibility throughout the modelling workflow.

### Software environment

The geospatial workflow was implemented primarily using QGIS (QGIS 3.24.3-Tisler), an open-source Geographic Information System used for spatial data preparation, raster processing, vector analysis, and cartographic visualization.

Spatial processing employed standard QGIS geoprocessing algorithms, including raster reprojection, raster alignment, raster calculations, zonal statistics, vector overlay operations, spatial intersections, and attribute calculations.

Statistical analyses, sensitivity analyses, and graphical outputs were performed in R (visualized using RStudio 2023.12.0 (R environment version 4.3.2, R Foundation for Statistical Computing Platform). The principal R packages used during the analysis included:

- sf – handling vector spatial data;

- terra – raster processing and spatial analysis;

- dplyr – attribute table manipulation;

- ggplot2 – statistical graphics and sensitivity plots.

No proprietary software or custom-developed algorithms were required. The proposed methodology relies exclusively on publicly available datasets and standard GIS operations, enabling straightforward implementation and independent reproduction by other researchers.

# COMPLETE GIS WORKFLOW FOR CALCULATING THE COMPOSITE EXPOSURE INDEX (CEI)

This supplementary material provides a detailed description of the complete geospatial workflow used to calculate the Composite Exposure Index (CEI). The workflow was implemented entirely in QGIS using standard raster and vector geoprocessing tools and consists of four major phases:

- preparation and harmonisation of input datasets;

- construction of the marine analytical grid;

- derivation of individual CEI components (traffic intensity, Natura 2000 coverage, and bathymetric susceptibility);

- calculation and classification of the final Composite Exposure Index.

The workflow was designed to ensure full methodological transparency and reproducibility and relies exclusively on publicly available datasets and open-source software.

All spatial analyses were performed in the ETRS89 / LAEA Europe projection (EPSG:3035) to ensure accurate calculations of areas, spatial overlaps, and zonal statistics. The analytical framework uses a regular 2 × 2 km equal-area grid as the basic spatial unit, enabling direct comparison among different environmental and anthropogenic indicators.

## PREPARATION OF THE INPUT DATASETS

The objective of the pre-processing stage was to harmonise all spatial datasets within a common spatial reference system and prepare them for grid-based spatial analysis.

Four publicly available datasets were imported into QGIS:

- IHO Adriatic Sea polygon, defining the marine study area;

- Natura 2000 marine protected areas, obtained from the European Environment Agency;

- AIS-derived recreational vessel density raster, representing boating intensity;

- GEBCO bathymetry raster, providing seabed depth information.

All datasets were transformed to the common coordinate reference system ETRS89 / LAEA Europe (EPSG:3035) to ensure accurate area calculations and spatial overlay.

Step 1. Import datasets

Input

| Dataset | Format |
| --- | --- |
| IHO Adriatic Sea boundary | Polygon |
| Natura 2000 | Polygon |
| AIS density | Raster |
| GEBCO bathymetry | Raster |

QGIS tools

- Layer → Add Vector Layer

- Layer → Add Raster Layer

Output:  Project containing four source layers.

Step 2. Seasonal aggregation of AIS vessel density data

Monthly AIS vessel density rasters downloaded from EMODnet were aggregated into seasonal mean rasters to represent typical recreational boating intensity during periods of high and low activity.

Two seasonal scenarios were considered:

- Summer: June–August

- Winter: January–March

Seasonal averages were calculated using the Cell Statistics tool in QGIS.

### QGIS tool

Processing Toolbox

→ Raster analysis

→ Cell statistics

Parameters

| Parameter | Summer scenario | Winter scenario |
| --- | --- | --- |
| Input rasters | June, July, August | January, February, March |
| Statistic | Mean | Mean |
| Output raster | Traffic_summer_mean.tif | Traffic_winter_mean.tif |

The resulting rasters represent the average seasonal recreational vessel density expressed as sailing hours per km².

Step 3. Reproject raster datasets

The AIS traffic raster and GEBCO bathymetry raster were reprojected into the common coordinate reference system.

QGIS tool

Raster -> Projections -> Warp (Reproject)

Parameters

| Parameter | Value |
| --- | --- |
| Target CRS | EPSG:3035 |
| Resampling | Bilinear |
| Output | GeoTIFF |

Output

- Traffic_summer_3035.tif

- Traffic_winter_3035.tif

- GEBCO_3035.tif

Raster and vector datasets must share the same coordinate system before overlay operations. The Natura 2000 polygon dataset downloaded from the European Environment Agency (EEA) was already provided in the ETRS89-extended / LAEA Europe projection (EPSG:3035). Therefore, no additional reprojection was required prior to spatial analysis.

## CONSTRUCTION OF THE MARINE ANALYTICAL GRID

Step 4. Create the analysis grid

QGIS tool

Processing Toolbox ->Vector creation -> Create Grid

Parameters

| Parameter | Value |
| --- | --- |
| Grid type | Rectangle |
| Cell size | 2000 × 2000 m |
| CRS | EPSG:3035 |
| Extent | IHO_Adriatic polygon |

Output: Grid_2km.gpkg

The grid provides identical spatial units for all subsequent analyses.

Step 5. Clip the grid to Adriatic Sea

QGIS tool

Processing -> Vector Overlay -> Clip ->Overlay layer -> IHO Adriatic Sea boundary

Input:  Grid_2km

Overlay:  IHO Adriatic

Output:   Grid_adr_clip.gpkg

Only cells intersecting the study area were retained.

Figure 1. Grid clipping workflow

Step 6. Calculation of marine cell area

After intersecting the regular grid with the IHO Adriatic Sea polygon, the actual marine area within each grid cell was calculated.

QGIS workflow

Right click layer

→ Open Attribute Table

→ Toggle Editing

→ Field Calculator

→ Create a new field

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | sea_area |
| Output field type | Decimal (double) |
| Expression | $area |

The resulting field stores the effective marine area (m²) within each grid cell.

## NATURA 2000 PROCESSING

Step 7. Clip Natura 2000

QGIS tool

Processing → Vector overlay → Clip

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | NaturaSite polygons |
| Overlay layer | IHO Adriatic Sea polygon |

Output: Natura_Adriatic.gpkg

Step 8. Dissolve Natura polygons

Several Natura 2000 designations overlap spatially (e.g. SAC and SPA). To avoid counting the same protected area multiple times, all polygons were dissolved into a single layer.

QGIS tool

Processing Toolbox -> Dissolve

Parameters: Dissolve all features

Output: Natura_dissolved.gpkg

Step 9. Natura 2000 grid intersection

The dissolved Natura 2000 polygon layer was intersected with the marine analytical grid to determine the spatial overlap between protected areas and individual grid cells.

QGIS tool

Processing → Vector overlay → Intersection

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | Grid_adr_sea.gpkg |
| Overlay layer | Natura_dissolved.gpkg |

Output: Grid_Natura_intersection.gpkg

Step 10. Calculate Natura area using Zonal Statistics

QGIS workflow

Right click layer: Grid_Natura_intersection.gpkg

→ Open Attribute Table

→ Toggle Editing

→ Field Calculator

→ Create a new field

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | nat_area |
| Output field type | Decimal (double) |
| Expression | $area |

Step 11. Calculate NaturaShare

Field Calculator

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | UdioNat_SeaArea |
| Output field type | Decimal (double) |
| Expression | "nat_area"/"sea_area" |

Cells without Natura 2000 returned NULL values.

These values were replaced by zero using Field Calculator:

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | UdioNat_SeaArea0 |
| Output field type | Decimal (double) |
| Expression | coalesce("UdioNat_SeaArea0",0) |

## BATHYMETRIC PROCESSING

Step 12. Calculate mean bathymetry

QGIS tool

Zonal Statistics

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | Zonal_statistic_Natura |
| Raster | GEBCO_3035.tif |
| Statistic | Mean |
| Prefix | depth_ |

Output: Zonal_Statistics_Bathymetry layer with new field depth_mean

Step 13. Convert bathymetric values

GEBCO bathymetric data represent underwater depths as negative elevation values relative to mean sea level. For easier interpretation and subsequent threshold classification, negative depth values were converted to positive water depths, while non-marine cells were assigned NULL values.

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | depth_sea |
| Output field type | Decimal (double) |
| Expression | if("depth_mean"<0,abs( "depth_mean" ), NULL ) |

Step 14. Derive the shallow-water indicator

The continuous depth values were transformed into the shallow-water indicator according to the threshold values defined in the main manuscript.

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | Shallow_ind |
| Output field type | Decimal (double) |
| Expression | CASE WHEN "depth_sea" IS NULL THEN 0 WHEN "depth_sea" <= 20 THEN 1 WHEN "depth_sea" >= 60 THEN 0 ELSE (60-"depth_sea")/(60-20) END |

## RECREATIONAL TRAFFIC PROCESSING

The complete workflow was implemented separately for two seasonal scenarios:

- Summer recreational boating exposure model

- Winter recreational boating exposure model.

All preprocessing steps related to bathymetry, Natura 2000 coverage, and analytical grid construction were identical in both scenarios. The only varying component was the seasonal AIS traffic layer.

Step 15. Calculate mean traffic density

QGIS tool

Zonal Statistics

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | Zonal_Statistics_Bathymetry |
| Raster | Traffic_summer_3035.tif |
| Statistic | Mean |
| Prefix | tr_ |

Output: Zonal_Statistics_Traffic layer with new field tr_mean

Step 16. Replace NULL values

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_clean |
| Output field type | Decimal (double) |
| Expression | coalesce("tr_mean",0) |

Step 17. Logarithmic transformation

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_log |
| Output field type | Decimal (double) |
| Expression | log10("tr_clean"+1) |

Step 18. Min–max normalisation

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_norm |
| Output field type | Decimal (double) |
| Expression | ("tr_log"-minimum)/("maximum"-minimum) |

The resulting values range from 0 to 1.

## COMPOSITE EXPOSURE INDEX

Step 19. Combine traffic and bathymetry

The normalised traffic density and the shallow-water indicator were combined using equal weights.

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | CEI |
| Output field type | Decimal (double) |
| Expression | "UdioNat_SeaArea0"*(0.5*"Shallow_ind" +0.5*"tr_norm") |

Step 20. Classify CEI

The continuous CEI values were classified into five exposure classes using the thresholds described in the manuscript.

Output

Exposure_Class

- No exposure

- Low

- Moderate

- High

- Critical

Field Calculator

CASE

WHEN  "CEI"  = 0 THEN 'No Risk'

WHEN  "CEI"  >0

AND  "CEI"  <=0.25 THEN 'Low'

WHEN  "CEI"  >0.25

AND  "CEI"  <=0.50 THEN 'Moderate'

WHEN  "CEI"  >0.50

AND  "CEI" <0.75 THEN 'High'

WHEN  "CEI"  >=0.75 THEN 'Critical'

END

### Seasonal replication

The entire workflow described in Sections V and VI was subsequently repeated using the winter recreational traffic raster (Traffic_winter_3035.tif) while keeping all other input layers and processing parameters unchanged. This produced an equivalent set of winter outputs (tr_mean, tr_log, tr_norm, CEI, and Exposure_Class) enabling direct seasonal comparison between summer and winter exposure patterns.

## Export for statistical analyses

The final seasonal grid layers were exported as .xlsx files and subsequently imported into R for statistical analyses and figure generation.

The complete R workflow is provided in Supplementary Material B.