Supplementary Material A

INTRODUCTION

This supplementary section provides a complete step-by-step description of the QGIS workflow used to produce the Composite Exposure Index (CEI). The protocol is intended to ensure methodological transparency and allow independent reproduction of all GIS procedures described in the manuscript.

This supplementary material provides a detailed description of the geospatial workflow developed to calculate the Composite Exposure Index (CEI) presented in the manuscript. The purpose of this document is to ensure methodological transparency and facilitate reproducibility of the proposed framework by providing a comprehensive description of all processing steps, input datasets, software environments, spatial analyses, and model calculations.

The CEI integrates three complementary spatial components representing different aspects of recreational boating-related environmental pressure:

- Traffic intensity, derived from multi-year Automatic Identification System (AIS) vessel density data

- Ecological sensitivity, represented by the proportional coverage of Natura 2000 marine protected areas within each analysis grid cell

- Bathymetric susceptibility, expressed through a shallow-water indicator derived from GEBCO bathymetric data.

Recreational vessel density and the shallow-water indicator are combined using scenario-based weighting coefficients, while the resulting value is scaled by the proportional Natura 2000 coverage within each marine grid cell. The analysis is implemented on a regular 1 × 1 km analytical grid clipped to the IHO Adriatic Sea boundary.

The complete workflow was implemented primarily in QGIS using standard geoprocessing tools, while statistical analyses and sensitivity analyses were performed in R. No proprietary algorithms or custom-developed software were required, enabling straightforward implementation by researchers and environmental managers using commonly available open-source software.

The complete R workflow, including scripts for data transformation, CEI calculations, classification procedures, sensitivity analyses, and figure production, is provided in Supplementary Material B.

DATA SOURCES AND SOFTWARE

INPUT DATASETS

IHO Adriatic Sea boundary

The study area was defined according to the official limits of the Adriatic Sea established by the International Hydrographic Organization (IHO, 1953). The digital representation of the Adriatic Sea boundary was obtained from the Marine Regions database maintained by the Flanders Marine Institute (VLIZ).

Source:

Marine Regions – Adriatic Sea (ID: 3314)

Dataset used: Marine Regions Gazetteer polygon layer.

Recreational vessel density (AIS)

Recreational boating activity was represented using vessel density products available through the European Marine Observation and Data Network (EMODnet Human Activities).

Monthly AIS vessel density rasters for the period 2017–2023, excluding 2020 because of COVID-19-related disruptions in maritime activity, were downloaded for the following categories:

- Sailing vessels (code 04)

- Pleasure craft (code 05)

The original raster resolution is 1 × 1 km, and values represent monthly sailing hours per km².

The data were aggregated into two seasonal periods: summer (June–September) and winter (December–March). Seasonal mean vessel density was calculated across the selected months and years for each analytical period. Values are expressed as monthly sailing hours per km² (h/km²/month).

Source:

EMODnet product catalogue

Natura 2000 marine protected areas

The Natura 2000 dataset was obtained from the European Environment Agency (EEA) as polygon shapefiles (version end 2024), representing the official boundaries of Special Areas of Conservation (SACs), Sites of Community Importance (SCIs), and Special Protection Areas (SPAs) within the European Natura 2000 network.

Only marine Natura 2000 sites located within the Adriatic Sea were retained for the analysis.

Source: European Environment Agency – Natura 2000 spatial dataset

Dataset format used:  ESRI Shapefile.

Bathymetric data

Bathymetric information was obtained from the General Bathymetric Chart of the Oceans (GEBCO) 2023 global bathymetric grid.

The dataset provides global bathymetry at a spatial resolution of 15 arc-seconds.

Source: GEBCO Web Services (WMS)

Table 1. Spatial datasets used in the CEI framework.

| Dataset | Provider | Format | Resolution | URL |
| --- | --- | --- | --- | --- |
| Adriatic Sea boundary | Marine Regions (VLIZ) | Polygon | – | Marine Regions – Adriatic Sea (ID: 3314) |
| Recreational vessel density (AIS) | EMODnet Human Activities | GeoTIFF | 1 km | EMODnet product catalogue |
| Natura 2000 polygons (version end 2024) | European Environment Agency | ESRI Shapefile (.shp) | Vector | European Environment Agency – Natura 2000 spatial dataset |
| Bathymetry | GEBCO | GeoTIFF | 15 arc-sec | GEBCO Web Services (WMS) |

SPATIAL REFERENCE SYSTEM

All spatial analyses were performed using the European Terrestrial Reference System 1989 – Lambert Azimuthal Equal Area projection (ETRS89 / LAEA Europe, EPSG:3035).

This equal-area projection was selected because it enables accurate calculations of area, spatial overlap, and zonal statistics across European marine regions while maintaining consistency among all spatial datasets.

Before analysis, all raster and vector datasets were transformed into this coordinate reference system. Raster datasets were additionally aligned to identical cell size, spatial extent, and pixel origin to ensure pixel-by-pixel compatibility throughout the modelling workflow.

Software environment

The geospatial workflow was implemented primarily using QGIS 3.24.3-Tisler, an open-source Geographic Information System used for spatial data preparation, raster processing, vector analysis, and cartographic visualization.

Spatial processing employed standard QGIS geoprocessing algorithms, including raster reprojection, raster alignment, raster calculations, zonal statistics, vector overlay operations, spatial intersections, and attribute calculations.

Statistical analyses, sensitivity analyses, and graphical outputs were performed in RStudio 2023.12.0 (R environment version 4.3.2, R Foundation for Statistical Computing Platform). The principal R packages used during the analysis included:

- sf – handling vector spatial data;

- terra – raster processing and spatial analysis;

- dplyr – attribute table manipulation;

- ggplot2 – statistical graphics and sensitivity plots.

No proprietary software or custom-developed algorithms were required. The proposed methodology relies exclusively on publicly available datasets and standard GIS operations, enabling straightforward implementation and independent reproduction by other researchers.

COMPLETE GIS WORKFLOW FOR CALCULATING THE COMPOSITE EXPOSURE INDEX (CEI)

PREPARATION OF THE INPUT DATASETS

The objective of the pre-processing stage was to harmonise all spatial datasets within a common spatial reference system and prepare them for grid-based spatial analysis.

Four publicly available datasets were imported into QGIS:

- IHO Adriatic Sea polygon, defining the marine study area;

- Natura 2000 marine protected areas, obtained from the European Environment Agency;

- AIS-derived recreational vessel density raster, representing boating intensity;

- GEBCO bathymetry raster, providing seabed depth information.

All datasets were transformed to the common coordinate reference system ETRS89 / LAEA Europe (EPSG:3035) to ensure accurate area calculations and spatial overlay.

Step 1. Seasonal aggregation of AIS vessel density data

Monthly AIS vessel density rasters downloaded from EMODnet were aggregated into seasonal mean rasters to represent typical recreational boating intensity during periods of high and low activity.

Two seasonal scenarios were considered:

- Summer: June– September

- Winter: December–March

Seasonal averages were calculated using the Cell Statistics tool in QGIS.

QGIS tool

Processing Toolbox

→ Raster analysis

→ Cell statistics

Parameters

| Parameter | Summer scenario | Winter scenario |
| --- | --- | --- |
| Input rasters | June, July, August, September | January, February, March |
| Years | 2017–2019, 2021–2023 | 2017–2019, 2021–2023 |
| Statistic | Mean | Mean |
| Output raster | Traffic_summer_mean.tif | Traffic_winter_mean.tif |

For each season, 48 monthly vessel-density rasters were included in the Cell Statistics calculation. The resulting rasters represent the average seasonal recreational vessel density expressed as h/km²/month.

Step 2. Import datasets

Input

| Dataset | Format |
| --- | --- |
| IHO Adriatic Sea boundary | Polygon |
| Natura 2000 | Polygon |
| summer mean AIS-derived recreational vessel density | Raster |
| winter mean AIS-derived recreational vessel density | Raster |
| GEBCO bathymetry | Raster |

QGIS tools

→ Layer → Add Vector Layer

→ Layer → Add Raster Layer

Output:  Project containing five source layers.

Step 3. Reproject raster datasets

Raster and vector datasets must share the same coordinate system before overlay operations. The seasonal AIS vessel-density rasters were already available in ETRS89 / LAEA Europe (EPSG:3035) and therefore required no further reprojection. The Natura 2000 dataset was also available in EPSG:3035. Therefore, no additional reprojection was required prior to spatial analysis.

The GEBCO bathymetry raster was reprojected into the common coordinate reference system.

QGIS tool

Raster -> Projections -> Warp (Reproject)

Parameters

| Parameter | Value |
| --- | --- |
| Target CRS | EPSG:3035 |
| Resampling | Bilinear |
| Output | GeoTIFF |

Output: GEBCO_3035.tif

CONSTRUCTION OF THE MARINE ANALYTICAL GRID

Step 4. Create the analysis grid

QGIS tool

Processing Toolbox ->Vector creation -> Create Grid

Parameters

| Parameter | Value |
| --- | --- |
| Grid type | Rectangle |
| Cell size | 1000 × 1000 m |
| CRS | EPSG:3035 |
| Extent | IHO_Adriatic polygon |

Output: Grid_1km.gpkg

The grid provides identical spatial units for all subsequent analyses.

Step 5. Clip the grid to Adriatic Sea

The regular grid was clipped to the IHO Adriatic Sea boundary.

QGIS tool

Processing -> Vector Overlay -> Clip ->Overlay layer -> IHO Adriatic Sea boundary

Parameters

| Parameter | Value |
| --- | --- |
| Input | Grid_2km |
| Overlay | IHO Adriatic |

Output:   Grid_adr_clip.gpkg

Only cells intersecting the study area were retained.

Figure 1. Grid clipping workflow

Step 6. Calculation of marine cell area

After clipping the regular grid with the IHO Adriatic Sea polygon, the actual marine area of each clipped cell was therefore calculated using $area and stored in the sea_area field. This value was used as the denominator in subsequent area-based calculations.

QGIS workflow

Right click layer

→ Open Attribute Table

→ Toggle Editing

→ Field Calculator

→ Create a new field

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | sea_area |
| Output field type | Decimal (double) |
| Expression | $area |

The resulting field stores the effective marine area (m²) within each grid cell.

BATHYMETRIC PROCESSING

Step 7. Reproject and prepare the GEBCO bathymetry raster

The original GEBCO bathymetry raster was reprojected to ETRS89 / LAEA Europe (EPSG:3035) using the QGIS Warp (Reproject) tool. Bilinear resampling was applied to the continuous bathymetric data.

QGIS tool

Raster → Projections → Warp (Reproject)

| Parameter | Value |
| --- | --- |

| Input raster | Original GEBCO bathymetry raster |
| --- | --- |
| Target CRS | EPSG:3035 |
| Resampling method | Bilinear |
| Output | GEBCO_3035_aligned.tif |

Step 8. Retain marine bathymetric values

The reprojected GEBCO raster (GEBCO_3035_aligned) contained negative values for seabed elevations below mean sea level and zero or positive values for terrestrial areas. To prevent terrestrial values from influencing bathymetric statistics in coastal grid cells, only negative values were retained for subsequent analysis, while non-marine values were assigned NoData.

QGIS tool

Raster  Raster Calculator

Expression

if(

"GEBCO_3035_aligned@1" < 0,

"GEBCO_3035_aligned@1",

NULL

)

Output: GEBCO_3035_sea_only.tif

These NoData cells were encoded using a value of −9999 and explicitly defined as NoData in the output GeoTIFF. Consequently, these cells were excluded from subsequent zonal-statistics calculations and did not contribute to mean bathymetry estimates.

QGIS tool

Processing Toolbox → GDAL → Raster conversion → Translate (Convert format)

| Parameter | Value |
| --- | --- |
| Input layer | GEBCO_3035_sea_only.tif |
| NoData value | -9999 |
| Output format | GeoTIFF |
| Output | GEBCO_3035_sea_only_nodata.tif |

Step 9. Calculate mean bathymetry

Mean bathymetry was calculated for the marine portion of each analytical grid cell using the grid previously clipped to the IHO Adriatic Sea boundary. This ensures that terrestrial portions of coastal cells do not influence the estimated mean depth.

QGIS tool

Processing Toolbox → Raster analysis → Zonal statistics

| Parameter | Value |
| --- | --- |
| Input layer | Grid_adr_sea.gpkg |
| Raster | GEBCO_3035_sea_only_nodata.tif |
| Statistic | Mean |
| Prefix | depth_ |

Output: depth_adr_grid_1000.gpkg layer with new field depth_mean.

GEBCO bathymetric data represent underwater depths as negative elevation values relative to mean sea level. For easier interpretation and subsequent threshold classification, negative depth values were converted to positive water depths. New field was calculated using Field Calculator.

| Parameter | Value |
| --- | --- |
| Output field name | depth_sea |
| Output field type | Decimal (double) |
| Expression | abs("depth_mean") |

Step 10. Derive the shallow-water indicator

The continuous depth values were transformed into the shallow-water indicator according to the threshold values defined in the main manuscript.

This value was derived using Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | Shallow_ind |
| Output field type | Decimal (double) |
| Expression | CASE WHEN "depth_sea" IS NULL THEN 0 WHEN "depth_sea" <= 20 THEN 1 WHEN "depth_sea" >= 60 THEN 0 ELSE (60-"depth_sea")/(60-20) END |

NATURA 2000 PROCESSING

Step 11. Clip Natura 2000

QGIS tool

Processing → Vector overlay → Clip

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | NaturaSite polygons |
| Overlay layer | IHO Adriatic Sea polygon |

Output: Natura_Adriatic.gpkg

Step 12. Dissolve Natura polygons

Several Natura 2000 designations overlap spatially (e.g. SAC and SPA). To avoid counting the same protected area multiple times, all polygons were dissolved into a single layer.

QGIS tool

Processing Toolbox -> Dissolve

Parameters: Dissolve all features

Output: Natura_dissolved.gpkg

Step 13. Natura 2000 grid intersection

The dissolved Natura 2000 polygon layer was intersected with the marine analytical grid to determine the spatial overlap between protected areas and individual grid cells.

QGIS tool

Processing → Vector overlay → Intersection

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | Grid_adr_sea.gpkg |
| Overlay layer | Natura_dissolved.gpkg |

Output: Grid_Natura_intersection.gpkg

Step 14. Calculate Natura area

QGIS workflow

Right click layer: Grid_Natura_intersection.gpkg

→ Open Attribute Table

→ Toggle Editing

→ Field Calculator

→ Create a new field

Parameters

| Parameter | Value |
| --- | --- |
| Output field name | nat_area |
| Output field type | Decimal (double) |
| Expression | $area |

Grid cells without Natura 2000 coverage contained NULL values in the nat_area field. These values were replaced with zero using the QGIS Field Calculator:

coalesce("nat_area", 0)

The resulting values were stored in a new field, nat0, representing Natura 2000 area within each grid cell, with zero assigned to cells without protected-area coverage.

Step 15. Join Natura 2000 attributes to the bathymetric grid

Following the calculation of mean bathymetry, Natura 2000 attributes were joined to the marine analytical grid using the unique grid-cell identifier (id). The bathymetric grid was retained as the primary analytical layer, while Natura 2000 attributes derived from the grid–Natura intersection were appended using a field-based join.

QGIS tool

Processing Toolbox → Vector general → Join attributes by field value

| Parameter | Value |
| --- | --- |
| Input layer | depth_adr_1000 |
| Table field | id |
| Input layer 2 | grid_nat_Intersection_dis2 |
| Table field 2 | id |
| Join type | Take attributes of the first matching feature only (one-to-one) |
| Output | depth_adr_nat_grid_1000.gpkg |

The resulting layer retained the complete marine analytical grid and the bathymetric attributes while adding the corresponding Natura 2000 information for each grid cell.

Step 16. Calculate NaturaShare

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | UdioNat_SeaArea |
| Output field type | Decimal (double) |
| Expression | "nat_area"/"sea_area" |

Cells without Natura 2000 returned NULL values.

These values were replaced by zero using Field Calculator:

| Parameter | Value |
| --- | --- |
| Output field name | UdioNat_SeaArea0 |
| Output field type | Decimal (double) |
| Expression | coalesce("UdioNat_SeaArea0",0) |

RECREATIONAL TRAFFIC PROCESSING

The complete workflow was implemented separately for two seasonal scenarios:

- Summer recreational boating exposure model

- Winter recreational boating exposure model.

All preprocessing steps related to bathymetry, Natura 2000 coverage, and analytical grid construction were identical in both scenarios. The only varying component was the seasonal AIS traffic layer.

Step 15. Calculate mean traffic density

QGIS tool

Zonal Statistics

Parameters

| Parameter | Value |
| --- | --- |
| Input layer | Zonal_Statistics_Bathymetry |
| Raster | Traffic_summer_3035.tif |
| Statistic | Mean |
| Prefix | tr_ |

Output: Zonal_Statistics_Traffic layer with new field tr_mean

Step 16. Replace NULL values

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_clean |
| Output field type | Decimal (double) |
| Expression | coalesce("tr_mean",0) |

Step 17. Logarithmic transformation

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_log |
| Output field type | Decimal (double) |
| Expression | log10("tr_clean"+1) |

Step 18. Min–max normalisation

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | tr_norm |
| Output field type | Decimal (double) |
| Expression | ("tr_log"-minimum)/("maximum"-minimum) |

The resulting values range from 0 to 1.

COMPOSITE EXPOSURE INDEX

Step 19. Combine traffic and bathymetry

The normalised traffic density and the shallow-water indicator were combined using equal weights.

Field Calculator

| Parameter | Value |
| --- | --- |
| Output field name | CEI |
| Output field type | Decimal (double) |
| Expression | "UdioNat_SeaArea0"*(0.5*"Shallow_ind" +0.5*"tr_norm") |

Step 20. Classify CEI

The continuous CEI values were classified into five exposure classes using the thresholds described in the manuscript.

Output

Exposure_Class

- No exposure

- Low

- Moderate

- High

- Critical

Field Calculator

CASE

WHEN  "CEI"  = 0 THEN 'No Risk'

WHEN  "CEI"  >0

AND  "CEI"  <=0.25 THEN 'Low'

WHEN  "CEI"  >0.25

AND  "CEI"  <=0.50 THEN 'Moderate'

WHEN  "CEI"  >0.50

AND  "CEI" <0.75 THEN 'High'

WHEN  "CEI"  >=0.75 THEN 'Critical'

END

Seasonal replication

The entire workflow described in Sections V and VI was subsequently repeated using the winter recreational traffic raster (Traffic_winter_3035.tif) while keeping all other input layers and processing parameters unchanged. This produced an equivalent set of winter outputs (tr_mean, tr_log, tr_norm, CEI, and Exposure_Class) enabling direct seasonal comparison between summer and winter exposure patterns.

Export for statistical analyses

The final seasonal grid layers were exported as .xlsx files and subsequently imported into R for statistical analyses and figure generation.

The complete R workflow is provided in Supplementary Material B.